

[image: image6.png]





Approved



       BULLETIN

          

Y4R 2026-3
3522-0000025 OM
Operation manual
[image: image1]










     Quantity of sheets        5












         Sheet        1
   GROUND: VK 3500-845 dd. 30.12.2025

Annotation
This service bulletin provides the following information:
-
the list of tractor maintenance operations after running-in has been clarified in terms of excluding the operation of changing the transmission oil after running-in the tractor;

-
a new list of tractor fuels and lubricants has been introduced.

Content of changes
In subsection 4.4. Tractor final assembly and run-in, clause 4.4.5 Maintenance after tractor run-in, the following changes shall me made:
The information available:

- Drain the transmission oil. Then clean the transmission mesh filter and magnetic catchers of the magnetic filter, replace the paper filter cartridges of the dual filter and clean the ring permanent magnets. Fill the transmission with fresh oil;

Shall be replaced with:

- clean the transmission mesh filter and magnetic catchers of the magnetic filter, filter cartridges of the dual filter and clean the ring permanent magnets.

Subsection 6.7 Fueling and lubrication of the tractor, shall be amended as follows:

6.7 Fueling and lubrication of the tractor
Table 6.7.1 shows the names and brands of fuels and lubricants used during operation and maintenance of the tractor, indicating their quantity and frequency of replacement.









            [image: image2.png]S
RU
D2

S 4







Sheet
Table 6.7.1 – List of fuels and lubricants for BELARUS-3522 tractors with WP10HG360E302 WEICHAI engine
	Item number
	Name of the assembly

units
	Qty. of assembly units, pcs.
	Name and designation of fuel and lubricant brands
	Weight (volume) of fuel and lubricants, refueled in the tractor per change, kg (dm3 )
	Change intervals for FLM, hours
	Remarks

	
	
	
	Main
	Duplicating
	Auxiliary
	Foreign-made
	
	
	

	1
	2
	3
	4
	5
	6
	7
	8
	9
	10

	1.1
	Fuel tank
	1
	At ambient temperature 0 ºС and above


	(640±2)


	Filled in every shift
	

	
	
	
	Fuel

Diesel

DT-L-K4,

DT-L-K5

Grade B

STB 1658-2015
	Not available
	Not available
	Fuel

diesel

EN 590:2025

with sulfur content

no more than

50 mg/kg (0.005%)


	
	
	

	
	
	
	At ambient temperatures of minus 5 ºС and above


	
	
	

	
	
	
	Diesel

Fuel

DT-L-K4,

DT-L-K5

Grade C

STB 1658-2015


	Not available
	Not available
	Fuel

diesel

EN 590:2025
with sulfur content no more than

50 mg/kg (0.005%)
	
	
	

	
	
	
	At ambient temperatures of minus 20 ºС and above


	
	
	

	
	
	
	Diesel 

Fuel

DT-Z-K5,

Class 0

Grade F
STB 1658-2015


	Not available
	Not available
	Fuel

diesel

EN 590:2025
with sulfur content no more than

50 mg/kg (0.005%)
	
	
	

	2 Oils

	2.1.1
	Engine oil crankcase

WEICHAI WP10HG360E302
	1
	According to the engine operating manual
	(30±0.5)

+ filter 

(1±0.l)
	First change 100 and next

500 or once every six months
	





















            [image: image3.png]S
RU
D2

S 4







Sheet

	Continuation of Table 6.7.1

	2.2
	Transmission housing (coupling clutch, gearbox and rear axle)
	1
	Transmission oils:

G - Special 

TO-4 10W;

MANNOL TO-4 Powertrain Oil SAE 10W;

Favorit TO-4 SAE 10W TU BY 690567607. 015-2013
	Transmission oils:

Mobiltrans HD 10W;​

Repsol Transmission

TO-4 10W;

Fuchs Titan UTTO

TO-4 10W;

ORLEN OIL Platinum Multi PTF 10W;

JASOL AGRICAT

SAE 10W;

Devon ТО-4 10W 

TU 0253-058-15301184-2015;

Hydraulic oil LUKOIL GEYSER ММ 10 (SAE 10W), TU 0253-013-79345251-2008
	Not available
	Not available
	(130±0.9)

between the marks on the dipstick
	1000

(250)
	

	2.3
	FDA housing

(coaxial planetary)
	1
	Transmission oils:

TAD -17i

GOST 23652-2023,

TM-5-18

GOST 17479.2-2015;

TM-5-18

TU BY 190106343. 024-2007
	Transmission oils:

Naftan TI5-1

TU RB 300220696.

007-2002;

LUKOIL GEAR

GL-5 80W-90

СТО 79345251-363-2023
	Not available
	HESSOL

BECHEM HYPOID

SAE 80W-90

API GL5/L4
	(18±0.1)
	1000
	

	2.4
	Wheel gear group housing of FDA

(coaxial planetary)

	2
	Transmission oils:

TAD -17i

GOST 23652-2023,

TM-5-18

GOST 17479.2-2015;

TM-5-18

TU BY 190106343. 024-2007
	Transmission oils:

Naftan TI5-1

TU BY 300220696.007-2002;

LUKOIL GEAR

GL-5 80W-90

СТО 79345251-363-2023
	Not available
	HESSOL

BECHEM

HYPOID

SAE 80W-90

API GL5/L4
	(6.4±0.06)
	500
	

	2.5
	Gear group of FPTO

(vertical )
	1
	Transmission oil is the same as in the transmission housings (coupling clutch, gearbox and rear axle)
	Not available
	Not available
	Not available
	(1.6±0.15)
	1000
	

	2.6
	Gear group of FPTO

(planetary)
	1
	Transmission oil is the same as in the transmission housings (coupling clutch, gearbox and rear axle)
	Not available
	Not available
	Not available
	(1,0±0.1 )
	1000
	

	2.7
	HLL and HSC tank with hydraulic units 
	1
	All season

hydraulic oils

Rosneft Gidrotec HLP 32, HLP 68;

VITTOL

HLP-32;

LUKOIL

Geyser

32ST, 68ST

Gazpromneft Hydraulic
HLP 32, HLP 68 1)
	All season 

hydraulic oils

AKC HLP32 DIN51524/2;

ТУ BY 190106343.032-2011

BECHEM

Staroil No32, No68;

ADDINOL

Hydraulikol

HLP 32;

HLP 68;

HYDROL

HLP 32,

HLP 68 1)
	Not available
	Not available
	(116.5±0.5)


	1000
	 

	2.8
	Air conditioner compressor
	1
	Oil

PAG 46
	Not available
	Not available
	Oil

PAG 46
	(0,06
±0.001)

	
	800, but at least once a year











            [image: image4.png]S
RU
D2

S 4







Sheet
	Continuation of Table 6.7.1

	1
	2
	3
	4
	5
	6
	7
	8
	9
	10

	3 Lubricants

	3.1
	Joint of steering hydraulic cylinder
	4
	
	Grease Litol-24

GOST 21150-2017
	Not available
	BECHEM LCP-GM
	0.05
±0.003
	250
	

	
	
	
	Grease MS-1000 TU 0254-003-45540231-99

	
	
	
	
	500
	

	3.2
	Steering rod joint
	2
	
	Grease Litol-24

GOST 21150-2017
	Not available
	Not available
	0.03

±0.003
	250


	

	
	
	
	Grease МS1600

TU 0254-035-45540231-2012
	Grease 

MS-1000

TU 0254-003-45540231-99​
	
	
	
	500
	

	3.3
	Bushing of RLL pivot shaft
	2
	
	Grease Litol-24
GOST 21150-2017
	Grease

Solidol C

GOST 4366-76

or

grease

solidol G

GOST 1033-79
	BECHEM LCP-GM;

Mobile Grease MP;

ISO-L-XDCIB2
	0.02

±0.001
	500


	

	
	
	
	Grease 

MS-1000

TU 0254-003-45540231-99
	
	
	
	
	1000
	

	3.4​
	RLL lifting rod yoke
	2
	
	Grease Litol-24

GOST 21150-2017
	Grease

Solidol C

GOST 4366-76

or

grease

solidol G

GOST 1033-79
	BECHEM LCP-GM;

Mobile Grease MP;

ISO-L-XDCIB2
	0.015
±0.003
	500

	

	
	
	
	Grease 

MS-1000

TU 0254-003-45540231-99
	
	
	
	
	1000
	

	3.5​
	Tow device “hook with shock absorber”
	2
	
	Grease Litol-24

GOST 21150-2017
	Grease

Solidol C

GOST 4366-76

or

grease

solidol G

GOST 1033-79
	BECHEM LCP-GM;

Mobile Grease MP;

ISO-L-XDCIB2
	0.02

±0.001
	500


	

	
	
	
	Grease 

MS-1000

TU 0254-003-45540231-99
	
	
	
	
	1000
	

	3.6
	Bushing of the pivot centre of FLL front rod
	2
	
	Grease Litol-24

GOST 21150-2017
	Grease

Solidol C

GOST 4366-76

or

grease

solidol G

GOST 1033-79
	BECHEM LCP-GM;

Mobile Grease MP;

ISO-L-XDCIB2
	0,1

±0.005
	500


	

	
	
	
	Grease 

MS-1000

TU 0254-003-45540231-99
	
	
	
	
	1000
	

	3.7
	RLL cylinder pin
	2
	
	Grease Litol-24

GOST 21150-2017
	Grease

Solidol C

GOST 4366-76

or

grease

solidol G

GOST 1033-79
	BECHEM LCP-GM;

Mobile Grease MP;

ISO-L-XDCIB2
	0.002

±0.001
	250


	

	
	
	
	Grease 

MS-1000

TU 0254-003-45540231-99
	
	
	
	
	500
	

	3.8​
	Bearing of FDA hoop
	2
	
	Grease Litol-24

GOST 21150-2017
	Grease

Solidol C

GOST 4366-76

or

grease

solidol G

GOST 1033-79
	BECHEM LCP-GM;
	0.08

±0.004
	250


	

	
	
	
	Grease 

MS-1000

TU 0254-003-45540231-99
	
	
	
	
	500
	











            [image: image5.png]S
RU
D2

S 4







Sheet
End of table 6.7.1

	1
	2
	3
	4
	5
	6
	7
	8
	9
	10

	3.9
	Bearings of FDA dual joint cross 
	2
	Grease

No158М

TU 38.301-40-25-94
	
	Not available
	Not available
	0.0112

±0.001
	One-time
	Not to be replenished during operation.

	3.10
	Bearing of the pivot axle of the FDA gear group
	4
	
	Grease Litol-24

GOST 21150-2017
	Grease

Solidol C

GOST 4366-76

or grease

solidol G

GOST 1033-79
	BECHEM LCP - GM
	0.12
± 0.006
	250


	

	
	
	
	Grease 

MS-1000

TU 0254-003-45540231-99
	
	
	
	
	500 


	

	3.11
	Fender swivel mechanism
	6
	
	Grease Litol-24

GOST 21150-2017
	Not available
	BECHEM LCP - GM
	0.02
± 0,001
	500


	

	
	
	
	Grease 

MS-1000

TU 0254-003-45540231-99
	
	
	
	
	1000


	

	3.12
	Control mechanism of the RLL  position sensor
	1
	
	Grease Litol-24

GOST 21150-2017
	Grease

Solidol C

GOST 4366-76

or grease

solidol G

GOST 1033-79
	BECHEM LCP-GM;

Mobile Grease MP;

ISO-L-XDCIB2
	0.1

±0.005
	500


	

	
	
	
	Grease 

MS-1000

TU 0254-003-45540231-99
	
	
	
	
	1000
	

	4 Special liquids

	4.1
	Clutch hydraulic drive reservoir and cylinders
	2
	Brake fluid

ROSDOT 4

TU 2451-004-

36732629-99
	Not available
	Not available
	DOT 3, DOT 4 (Germany)
	(0.8±0.2)
	1000 

(8-10)​


	

	4.2
	Brake hydraulic drive reservoir
and cylinders
	3
	Brake fluid

ROSDOT 4

TU 2451-004-

36732629-99
	Not available
	Not available
	DOT3; DOT4 
(Germany)
	(1.2±0.3)
	1000 (500)


	

	4.3
	Сooling system of the

WEICHAI WP10HG360E302 engine
	1
	Antifreeze

FELIX CARBOX,

FELIX CARBOX SQ
	Not available
	Not available
	According to the engine operating manual
	(60,0
±0.5)
	2000 or once a year
	

	4.4
	Air conditioning system
	2
	Freon R134a
	Not available
	Not available
	Freon R134a
	1.7±0.02
	800, but once a year
	

	______________________________________________________________________________________________

1) Hydraulic oils HLP 68, No.68, 68CT are used for tractors supplied to countries with hot climate.


Bulletin


Y4R 2026-3





TRACTOR MODEL�
3522 with WEICHAI engine�
�






Bulletin


Y4R 2026-3








 2





Bulletin 


Y4R 2026-3








 3





Bulletin


Y4R 2026-3








 4





Bulletin


Y4R 2026-3








 5








